# Adaptive Filtering for Clutter Mitigation in Radar Systems

## ### A-GILE Chat Conversation — Mircea Grecu / Leonardo DRS

## Q: Can adaptive filtering be useful in clutter mitigation in radar systems?

I have multi-channel spectrum data for potential detections and would like to process them. Most of these detections are false. The channels are observations collected by various subarrays of the same radar. In principle the observations in the Doppler domain are available for the entire beam, but it is preferable to do point (one per detection) calculations using only the measurement associated with that range bin, as the computational load is much lower and the data is already available in the existing structures.

## A: Adaptive Filtering for Clutter Mitigation

Yes — adaptive filtering is well-suited to this problem, and the single-point-per-detection constraint maps cleanly onto a classical framework called Space-Time Adaptive Processing (STAP), or in the reduced case, a spatial-only adaptive filter applied per range bin in the Doppler domain.

### Why Adaptive Filtering Works Here

Clutter tends to occupy a structured subspace in the joint spatial (subarray channel) × Doppler space. The key property adaptive methods exploit is that clutter is correlated across channels — it arrives from the same direction/range and appears coherently across subarrays — while a true target plus noise has a different spatial signature.

Operating per detection in the Doppler domain (one snapshot per candidate detection, one Doppler bin at a time) corresponds to working in the spatial domain only at each Doppler frequency. This is sometimes called Doppler-bin-by-Doppler-bin STAP or simply a spatial adaptive filter.

### The Core Framework

#### Data Model per Doppler bin

For a candidate detection at range bin r and Doppler bin k, stack the M subarray observations into a vector:

xr,k∈CM

The signal model is:

xr,k=α s+cr,k+nr,k

where: - α is the (unknown) complex target amplitude - s∈CM is the spatial steering vector (known from array geometry + look direction) - cr,k is the clutter component (correlated across channels) - nr,k is thermal noise (uncorrelated)

#### Adaptive Filter (MVDR / Matched Subspace)

The optimal linear filter is the Minimum Variance Distortionless Response (MVDR) beamformer:

w*=R-1ssHR-1s

where R=E[xxH] is the spatial covariance matrix of interference-plus-noise at that Doppler bin.

The filter output is:

y=wHxr,k

This maximises SINR by nulling clutter directions while preserving the target response along s.

### The Covariance Estimation Problem

| Strategy | Description | Tradeoffs |
| --- | --- | --- |
| Local range training | Use nearby range bins at the same Doppler bin | Standard; requires homogeneous clutter; guard cells needed near detection |
| Doppler-bin neighbours | Use adjacent Doppler bins at same range | Valid if clutter spectrum is locally smooth |
| Diagonal loading | R+δI with modest δ | Robustifies against small-sample estimation; always recommended |
| Persisted clutter map | Use clutter covariance accumulated over prior scans | Stable, scan-to-scan consistent |
| Sample covariance from beam | Use all range bins in beam | Not range-localised but stable; good fallback |

For M channels you need at least 2M independent training samples (Reed-Mallet-Brennan rule) for a 3 dB loss relative to the known-R case.

### Practical Detection Statistic

The standard detector is the Adaptive Matched Filter (AMF) or GLRT:

Λ=|sHR-1x|2sHR-1s≷η

## Q:

The clutter is not constant with range. Some of my channels are auxiliary channels. They are characterized by lower gain and broader beams (only one element). How are the auxiliary channel measurements used to mitigate clutter? I think this is a different paradigm than STAP (although related).

## A: Sidelobe Cancellation (SLC)

You are right — this is Sidelobe Cancellation (SLC), sometimes called Adaptive Sidelobe Cancellation or in its generalised form, Generalized Sidelobe Canceller (GSC).

### The Key Distinction

| Aspect | STAP / MVDR | Sidelobe Cancellation |
| --- | --- | --- |
| All channels | Equal-role, coherent aperture | Main channel is primary; aux channels are references |
| Steering vector | Applied across all channels jointly | Applied only to main channel; aux channels are unconstrained |
| Clutter model | Clutter occupies a subspace in joint space | Aux channels observe clutter without (or with minimal) target |
| Target preservation | Via distortionless constraint on s | Implicit — aux channels don’t see the target |
| Covariance needed | Full M×M across all channels | Cross-correlation between main and aux channels only |

### The Signal Model

Let: - y0 = main channel output at a given Doppler bin - ya∈CNa = auxiliary channel outputs (Na aux elements)

The main channel contains:

y0=αs0+c0+n0

The auxiliary channels observe:

ya=ca+na

with no target term (by assumption).

### The Canceller Output

The adaptive sidelobe canceller forms:

z=y0-wHya

The optimal weights are:

w*=Raa-1 ra0

where: - Raa=E[yayaH] — aux channel covariance (Na×Na) - ra0=E[ya y0*] — cross-correlation between aux channels and main channel

The minimum output power is:

Pmin=σ02-ra0HRaa-1ra0

### Practical Considerations

- For Na=1 auxiliary channel, Raa is a scalar (1 real value) and ra0 is a vector of length equal to the number of main channels.

- Pre-compute and store the optimal weight vector w*=Raa-1ra0 directly (15 complex values per cell for a 15-main-channel system), updated incrementally each scan.

- Use Sherman-Morrison for rank-1 covariance updates:

Raa,n+1-1=11-λRaa,n-1-λ Raa,n-1yayaHRaa,n-1(1-λ)+λ yaHRaa,n-1ya

## Q: To estimate Raa and ra0 in time, I need to store them. For a 5 channel system (out of which one is the auxiliary) I need to store 20 values. Is that correct?

## A: Storage Count

With 4 main channels and 1 auxiliary channel:

- Raa: scalar — 1 real value

- ra0: 4 complex values — 8 real values

- Total: 9 real values per range bin

The 20-value count would apply if there were 4 auxiliary channels.

## Q: Yes, I messed up the channel numbers. But for a radar with 15 guard channels and 4 aux, I need 4×4+4×15=4×19 values. Is that correct? If yes, just say yes.

## A: Yes.

## Q: My current clutter map stores 101 Doppler bin values per range and per direction. The SLC would roughly double for the 15+4 channel radar. I am not sure that is doable.

## A: Storage Analysis

| Quantity | Current CM | SLC addition |
| --- | --- | --- |
| Per range bin, per direction | 101 complex values | 4×19=76 complex values |
| Ratio | 1× | ~0.75× |

### Ways to Reduce the SLC Storage Burden

- Don’t store per Doppler bin — accumulate over the clutter Doppler region only, storing a single set of 76 values per range-direction cell.

- Exploit Hermitian structure — Raa is Hermitian; store only the upper triangle: 4×52=10 complex values instead of 16.

- Pre-compute and store Raa-1ra0 directly — only 15 complex values per cell, eliminating matrix inversion at detection time.

## Q: Can you explain how to update Raa-1ra0 without storing it?

## A: You Cannot — But Here Are the Alternatives

To update w*=Raa-1ra0 incrementally without storing Raa and ra0 separately, you need both to apply the update correctly.

Option 1 — Store Raa-1 and ra0 separately, update via Sherman-Morrison. Total storage: 25 complex values per cell.

Option 2 — LMS adaptive update on w* directly:

wn+1*=wn*+μ en ya,n

where en=y0,n-wn*Hya,n. Stores only w* (15 complex values), but convergence is slower.

Practical recommendation: Option 1 with Sherman-Morrison — 25 complex values vs. your current 101.

## Q: But let’s talk a bit more of the adaptive filtering you started with. How do you define the steering direction s?

## A: The Steering Vector

s∈CM is the array response vector to a target arriving from the beam steering direction θ0:

sm=ej2πλdmsinθ0, m=1,…,M

where dm is the phase centre position of subarray m.

At a given range bin r and Doppler bin k, the M subarray observations form a snapshot xr,k∈CM. A target at angle θ0 with complex amplitude α contributes:

xr,ktarget=α⋅s(θ0)

So s tells you what a true target looks like in the M-dimensional observation space. The MVDR filter places a null toward clutter directions while enforcing unit response toward θ0 — that is the distortionless constraint wHs=1.

## Q: How do you make this efficient? Do you explicitly calculate the covariance and apply the textbook formula? Or are you using equivalent more efficient formulations?

## A: Efficient Formulations

### The Textbook Formula and Its Cost

w*=R-1ssHR-1s

Done naively: O(M2N) to form R, O(M3) to invert, O(M2) to apply.

### More Efficient Formulations

1. Never invert R — solve a linear system instead. Use Cholesky factorisation (R=LLH), cost O(M3/3) — about 3× cheaper than general inversion.

2. Precompute R-1s when s is fixed per beam. Since s is the same for all range bins in a beam, solve once per beam dwell. Per-detection cost reduces to O(M).

3. Eigendecomposition / Subspace Methods. If clutter occupies a low-rank subspace (rank r≪M):

R-1=UcΛc-1-1σ2IUcH+1σ2I

Cost drops from O(M2) to O(Mr).

4. Recursive update via Sherman-Morrison:

Rn-1=11-λRn-1-1-λ Rn-1-1xnxnHRn-1-1(1-λ)+λ xnHRn-1-1xn

Cost per update: O(M2) — no factorisation needed each scan.

### Recommended Architecture

| Step | When | Cost |
| --- | --- | --- |
| Accumulate R via recursive update | Each scan, per range bin | O(M2)​ |
| Cholesky factor Rδ | Once per beam dwell | O(M3/3)​ |
| Solve for u=R-1s | Once per beam dwell | O(M2)​ |
| Compute Λ=¦uHx¦2uHs per detection | Per detection | O(M)​ |

## Q: How do you get UcΛc in the eigendecomposition approach?

## A: Obtaining the Eigendecomposition

Option 1 — Full EVD of the sample covariance (batch). Use a standard Hermitian eigendecomposition. Threshold to identify the clutter subspace:

λ1≥λ2≥…≥λr≫λr+1≈…≈λM≈σ2

Option 2 — SVD of the data matrix (preferred in practice). Work directly with the data matrix X∈CM×N:

X=UΣVH⟹R=UΣ2NUH

Numerically more stable than forming R explicitly.

Option 3 — PAST subspace tracking (recursive). For scan-to-scan updates:

hn=Un-1Hxn

Un=Un-1+1∥hn∥2(xn-Un-1hn)hnH

Cost per update: O(Mr).

For M=15, the full EVD costs roughly 153≈3400 flops — essentially free on modern hardware. Subspace tracking only pays off meaningfully for large M or very high update rates.

## Q: Do you have to calculate one R per Doppler bin?

## A: One R Per Doppler Bin Is Not Always Necessary

### The Clutter Subspace Structure

For a side-looking or forward-looking radar, the clutter subspace Uc may be largely the same across Doppler bins (same physical scatterers, same spatial directions). Only the eigenvalues Λc change significantly. So a reasonable approximation is:

R(k)≈UcΛc(k)UcH+σ2I

### Practical Strategies

| Doppler region | What R(k) looks like | Recommended approach |
| --- | --- | --- |
| Near zero Doppler | High clutter power, full-rank interference | Per-bin or clutter-region covariance |
| Moderate Doppler | Clutter falling off | Shared Uc, scaled eigenvalues |
| High Doppler (target region) | Clutter negligible, noise dominated | R≈σ2I — no adaptive filtering needed |

Strategy 2 — a single covariance estimated from the clutter Doppler region — gives 90% of the benefit at a fraction of the cost.

## Q:

Given the limitations of SLC in my problem, I actually want to investigate the adaptive processing approach. Could you extend the SLC model to include direction-dependent clutter?

## A: Complete Simulation Model

### 1. Geometry and Notation

M=5 channels, carrier wavelength λ=c/fc, antenna frame with x boresight, y lateral (right), z vertical (up).

### 2. Subarray Phase Centres and Aperture

pm=1|Sm|k∈Sm​pk∈R3

Dm=21|Sm|k∈Sm​∥pk-pm∥2

### 3. Direction Vectors

u(ϕ,θ)=cosθcosϕcosθsinϕsinθ

### 4. Steering Vector

ϕm(u)=2πλ uTpm,  s(u)=ejϕ1(u)⋮ejϕM(u)

### 5. Gaussian Beam Pattern

σm=λπDmDm>0∞Dm=0,  Gm(δθi)=exp​-δθi22σm2Dm>01Dm=0

### 6. Target Signal

fd=2vλ,  α∼CN(0,Ptgt)

xtgt(n)=α⋅ej2πfdnTPRI⋅G(δθtgt)⊙s(utgt)

### 7. Clutter Model — Rice Compound

Each scatterer i has a compound amplitude:

ampc,i(t)=m2 X1,i2+X2,i2(t)m2+1

where: - X1,i∼Weibull(kw,λw) — persistent, frozen for all scans - X2,i(t)∼N(0,σ2) — fluctuating, redrawn each scan - σ=λwΓ(1+2/kw) — preserves total power (E[X2]=E[X12] independent of m)

Scan-to-scan amplitude correlation:

Corr​amp(t), amp(t')≈m2m2+1

Scatterer angular positions (drawn once, persistent):

ϕc,i=ϕc+δϕi, δϕi∼N(0,σaz2)

θc,i=θc+δθiel, δθiel∼N(0,σel2)

Scatterer phase (persistent — critical for spatial covariance):

ψi∼U(0,2π), drawn once and held fixed across scans

Total clutter snapshot:

xclut=i=1Ncampc,i(t)⋅ejψi⋅G(δθi)⊙s(uc,i)

### 8. Thermal Noise

xnoise(n)∼CN(0, σn2IM)

### 9. Total Received Pulse Train

X=xtgtdT+xclut1T+Xnoise

where d=[1,ej2πfdTPRI,…,ej2πfd(N-1)TPRI]T.

### 10. Doppler Processing

Window amplitude-normalised to preserve coherent gain:

w(n)=w(n)1Nn'=0N-1w(n')

Windowed DFT:

Xm(k)=n=0N-1w(n) xm(n) e-j2πkn/N

### 11. MVDR Filter

Covariance estimated from the clutter Doppler region, channels 1–4:

R=1Nscanst​x(k0,t) xH(k0,t)+δI

w*=R-1ssHR-1s,  Z(k)=wHx(k) ∀ k

## Q: SLC vs Adaptive Filtering — Conceptual Comparison

SLC is fundamentally a subtraction operation. With distributed clutter and an omnidirectional aux antenna the reference is a spatial average with no angular discrimination. The value of SLC in such cases is the identification of a point target — with multiple omnidirectional aux elements you gain spatial diversity to check whether a detection is consistent with a point-like scatterer (target) or a spatially distributed one (clutter).

MVDR / Adaptive Filtering uses all available spatial degrees of freedom to reshape the beam pattern adaptively around the interference — a fundamentally more powerful operation than reference subtraction.

|  | SLC | MVDR |
| --- | --- | --- |
| Mechanism | Subtract a reference | Invert the interference covariance |
| What it exploits | Correlation between aux and main | Full spatial structure across all channels |
| Degrees of freedom | One per aux channel | M-1 across all channels |
| Clutter model needed | Implicit, via cross-correlation | Explicit, via R |
| Target protection | By construction (aux sees no target) | Via distortionless constraint on s |

## Q: Why does the zero-Doppler covariance apply to all Doppler bins?

## A: Separability Assumption

The argument rests on a separability assumption:

R(k)=Pc(k)⋅Rspatial

Then R-1(k)=1Pc(k)Rspatial-1 and the MVDR weight vector:

w*(k)=R-1(k)ssHR-1(k)s=Rspatial-1ssHRspatial-1s

is identical for all k because Pc(k) cancels.

### When Separability Holds

- Clutter scatterers are stationary

- Doppler spectrum shape is the same for all scatterers

- No angle-Doppler coupling (ground-based radar)

### When It Breaks Down

- Airborne radar — angle-Doppler coupling: fd(u)=2λvplatform⋅u; full STAP is required

- Non-uniform internal clutter motion — different scatterers have different velocity distributions

- High Doppler bins — noise-dominated; R≈σ2I, no filtering needed

- Near zero-Doppler with slow target — target self-nulling risk

## Q: How do you extend it to the full STAP with two Doppler sub-domains?

## A: Full STAP Extension

### The STAP Observation Vector

Stack spatial snapshots from both sub-domains:

xST=x(klow)x(khigh)∈C8

The covariance is:

RST=R(klow,klow)R(klow,khigh)R(khigh,klow)R(khigh,khigh)∈C8×8

The off-diagonal blocks capture cross-Doppler spatial correlation — non-zero whenever clutter has correlated energy across Doppler bins.

### The STAP Steering Vector

The Doppler response of a target with Doppler fd at bin k after windowed DFT:

d(k,fd)=n=0N-1w(n) ej2πfdnTPRIe-j2πkn/N

The STAP steering vector is the Kronecker product of Doppler and spatial steering vectors:

sST=d(klow, fd)d(khigh, fd)⊗sspatial∈C8

### Clutter vs Target in the Stacked Space

|  | d(klow,fd)​ | d(khigh,fd)​ |
| --- | --- | --- |
| Zero-Doppler clutter | Large | Small (sidelobe leakage only) |
| Slow target | Large | Small — similar to clutter |
| Fast target | Small | Large — very different from clutter |
| General target | ejϕlow​ | ejϕhigh — specific phase ratio |

### MVDR in the Stacked Space

wST*=RST-1sSTsSTHRST-1sST

Λ=|sSTHRST-1xST|2sSTHRST-1sST≷η

## Q:

My previous steering vector was only spatial because velocity was 0 and exp(j2πfdkTPRI) was 1. But in the more general case it is a complex number that is not 1. Do not further explain. Just confirm if true.

## A: Confirmed.

End of conversation export.